This assignment requires you to perform packet capture using Wireshark. The capture should be at least 7 minutes in length, and you will only need one capture to work with. You will then create a report that shows the following in a Word format, supported with highlighted screenshots and detailed explanations explicitly from the capture:

- MAC of your device and MAC of your Gateway: You can find the MAC address of your device and gateway in the Ethernet frames of the captured packets. The source MAC address of the Ethernet frame is the MAC address of the device that sent the packet, while the destination MAC address is the MAC address of the device that should receive the packet. The MAC address of your device should be the source MAC address of the Ethernet frames sent by your device, while the MAC address of your gateway should be the destination MAC address of the Ethernet frames sent by your device 1.

- ARP messages (complete request): ARP messages are used to resolve IP addresses to MAC addresses. When a device wants to communicate with another device on a local network, it sends an ARP request message to the network asking for the MAC address of the device with the specified IP address. The device with the specified IP address responds with an ARP reply message containing its MAC address 2. You can find ARP messages in the captured packets by applying a filter for ARP packets. The ARP request message contains the IP address of the device that is being queried, while the ARP reply message contains the MAC address of the device that is responding 3.

- HTTP request and message details (full request details from start to finish): HTTP is a protocol used for data transfer over the internet. HTTP stands for HyperText Transfer Protocol. URLs for HTTP begin with “http://”. HTTP is considered to be unsecure 456. You can find HTTP requests and message details in the captured packets by applying a filter for HTTP packets. The HTTP request message contains the details of the request being made, while the HTTP response message contains the details of the response being sent 7.

- Compare results against HTTPS: HTTPS is a secure version of HTTP that uses Transport Layer Security (TLS) to encrypt the data exchanged between the browser and server, making it more secure than HTTP 8910. You can compare the results of HTTP and HTTPS by analyzing the captured packets and comparing the differences in the protocols used.

- HTTPS request and message details (full request details from start to finish): HTTPS is a secure version of HTTP that uses Transport Layer Security (TLS) to encrypt the data exchanged between the browser and server, making it more secure than HTTP 8910. You can find HTTPS requests and message details in the captured packets by applying a filter for HTTPS packets. The HTTPS request message contains the details of the request being made, while the HTTPS response message contains the details of the response being sent .

- Using WinSCP, use the below details to download a file over FTP, show the results in the capture and explain (full details of one file), does this need to be secured? how?

  - FTP URL: ftp.dlptest.com or ftp://ftp.dlptest.com/

  - FTP User: dlpuser

  - Password: rNrKYTX9g7z3RgJRmxWuGHbeu

You can use WinSCP to download a file over FTP. To do this, you will need to enter the FTP URL, username, and password into WinSCP. Once you have connected to the FTP server, you can browse the files on the server and download the file you need. You can then analyze the captured packets to show the results of the file transfer and explain the full details of one file.

FTP is considered to be unsecure because it sends data in plain text, including usernames, passwords, and file contents. To secure FTP, you can use FTPS (FTP over SSL) or SFTP (SSH File Transfer Protocol) .

- Broadcast messages: Broadcast messages are messages that are sent to all devices on a network. You can find broadcast messages in the captured packets by applying a filter for broadcast packets. Broadcast messages are used for various purposes, such as network discovery, service announcements, and time synchronization .

- DNS messages (complete request), how to secure this?: DNS stands for Domain Name System. It is a protocol used to translate domain names into IP addresses. DNS messages are used to query DNS servers for the IP address of a domain name. You can find DNS messages in the captured packets by applying a filter for DNS packets. The DNS request message contains the domain name being queried, while the DNS response message contains the IP address of the domain name .

To secure DNS, you can